Servo motor Code
import time

import os

PWM_CHIP    = "/sys/class/pwm/pwmchip13"

PWM_CHANNEL = 0

PWM_PATH    = f"{PWM_CHIP}/pwm{PWM_CHANNEL}"

PERIOD_NS   = 20_000_000

def pwm_write(path, val):

with open(path, "w") as f:

f.write(str(val))

def angle_to_duty(angle):

pulse_ms = 0.5 + (angle / 180.0) * 2.0

return int(pulse_ms * 1_000_000)

# Setup

if not os.path.exists(PWM_PATH):

pwm_write(f"{PWM_CHIP}/export", PWM_CHANNEL)

time.sleep(0.2)

pwm_write(f"{PWM_PATH}/period",     PERIOD_NS)

pwm_write(f"{PWM_PATH}/polarity",   "normal")   # ← FIX added

pwm_write(f"{PWM_PATH}/duty_cycle", angle_to_duty(90))

pwm_write(f"{PWM_PATH}/enable",     1)

print("Hardware PWM ready ✓")

def move_servo(angle):

print(f"→ {angle}°")

pwm_write(f"{PWM_PATH}/duty_cycle", angle_to_duty(angle))

time.sleep(0.6)

pwm_write(f"{PWM_PATH}/duty_cycle", 0)  # silence = no vibration

try:

while True:

for angle in range(0, 181, 20):

move_servo(angle)

time.sleep(1)

for angle in range(180, -1, -20):

move_servo(angle)

time.sleep(1)

except KeyboardInterrupt:

print("\nStopped")

finally:

pwm_write(f"{PWM_PATH}/enable",   0)

pwm_write(f"{PWM_CHIP}/unexport", PWM_CHANNEL)

print("PWM released ✓")



HLK LD 2451 codes
Arduino Code :
import serial

import serial.tools.list_ports

import tkinter as tk

import matplotlib.pyplot as plt

import matplotlib.gridspec as gridspec

from matplotlib.backends.backend_tkagg import FigureCanvasTkAgg

from matplotlib.animation import FuncAnimation

import collections

# ─────────────────────────────────────────────

# PORT SETUP

# ─────────────────────────────────────────────

PORT = "/dev/radar"   # fixed via udev rule

BAUD = 115200

try:

ser = serial.Serial(PORT, BAUD, timeout=1)

print(f"Connected: {PORT}")

except Exception:

# Fallback: auto detect

ser = None

for p in serial.tools.list_ports.comports():

if 'ACM' in p.device or 'USB' in p.device:

try:

ser = serial.Serial(p.device, BAUD, timeout=1)

PORT = p.device

print(f"Fallback port: {PORT}")

break

except Exception:

continue

if ser is None:

print("ERROR: No Arduino port found!")

exit(1)

# ─────────────────────────────────────────────

# DATA BUFFERS

# ─────────────────────────────────────────────

MAX_POINTS = 100

dist_data  = collections.deque([0] * MAX_POINTS, maxlen=MAX_POINTS)

speed_data = collections.deque([0] * MAX_POINTS, maxlen=MAX_POINTS)

snr_data   = collections.deque([0] * MAX_POINTS, maxlen=MAX_POINTS)

# ─────────────────────────────────────────────

# WINDOW SETUP

# ─────────────────────────────────────────────

root = tk.Tk()

root.title("HLK-LD2451 Radar Monitor")

root.configure(bg="#0a0a0a")

# Left half of screen

sw = root.winfo_screenwidth()

sh = root.winfo_screenheight()

root.geometry(f"{sw // 2}x{sh}+0+0")

# ─────────────────────────────────────────────

# INFO BOX

# ─────────────────────────────────────────────

info_frame = tk.Frame(root, bg="#0a0a0a", pady=6)

info_frame.pack(fill="x", padx=12)

tk.Label(info_frame,

text="━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 10)).pack(anchor="w")

tk.Label(info_frame,

text="  HLK-LD2451  RADAR  MONITOR",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 13, "bold")).pack(anchor="w")

tk.Label(info_frame,

text="━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 10)).pack(anchor="w")

# Field definitions: (label, color)

fields = [

("Alarm",     "#ffffff"),

("Angle",     "#00cfff"),

("Distance",  "#ffff00"),

("Direction", "#ff4444"),

("Speed",     "#ff9900"),

("SNR",       "#cc88ff"),

]

labels = {}

for field, color in fields:

row = tk.Frame(info_frame, bg="#0a0a0a")

row.pack(anchor="w", pady=2)

tk.Label(row,

text=f"  {field:<12}: ",

bg="#0a0a0a", fg="#888888",

font=("Courier", 12)).pack(side="left")

labels[field] = tk.Label(row,

text="---",

bg="#0a0a0a", fg=color,

font=("Courier", 12, "bold"))

labels[field].pack(side="left")

tk.Label(info_frame,

text="━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 10)).pack(anchor="w")

# Status bar

status_var = tk.StringVar(value="  Waiting for target...")

tk.Label(info_frame,

textvariable=status_var,

bg="#0a0a0a", fg="#555555",

font=("Courier", 9)).pack(anchor="w", pady=3)

# ─────────────────────────────────────────────

# GRAPHS

# ─────────────────────────────────────────────

fig = plt.Figure(figsize=(sw / 2 / 100, sh * 0.52 / 100), dpi=100)

fig.patch.set_facecolor("#0a0a0a")

gs = gridspec.GridSpec(3, 1, figure=fig,

hspace=0.65,

top=0.93, bottom=0.08)

def make_axis(gs_slot, title, ylabel, color, ylim):

ax = fig.add_subplot(gs_slot)

ax.set_facecolor("#111111")

ax.set_title(title, color=color, fontsize=8, pad=3)

ax.set_ylabel(ylabel, color=color, fontsize=7)

ax.tick_params(colors="#444444", labelsize=6)

ax.set_ylim(*ylim)

ax.set_xlim(0, MAX_POINTS)

ax.grid(True, color="#1e1e1e", linewidth=0.5)

for sp in ax.spines.values():

sp.set_color("#2a2a2a")

line, = ax.plot([], [], color=color, linewidth=1.3)

return ax, line

ax1, line_dist  = make_axis(gs[0], "Distance",       "m",    "#ffff00", (0, 50))

ax2, line_speed = make_axis(gs[1], "Speed",           "km/h", "#ff9900", (0, 30))

ax3, line_snr   = make_axis(gs[2], "Signal Strength", "SNR",  "#cc88ff", (0, 255))

canvas = FigureCanvasTkAgg(fig, master=root)

canvas.get_tk_widget().pack(fill="both", expand=True, padx=6, pady=4)

# ─────────────────────────────────────────────

# UPDATE FUNCTION

# ─────────────────────────────────────────────

def update(_):

try:

updated = False

while ser.in_waiting:

raw   = ser.readline().decode(errors="ignore").strip()

parts = raw.split(",")

if len(parts) != 6:

continue

alarm, angle, distance, direction, speed, snr = parts

try:

alarm_int = int(alarm)

dist_f    = float(distance)

speed_f   = float(speed)

snr_f     = float(snr)

except ValueError:

continue

if alarm_int > 1:

continue

# Update info labels

dir_color = "#ff4444" if direction == "Approaching" else "#00ff88"

labels["Alarm"].config(text=alarm)

labels["Angle"].config(text=f"{angle} deg")

labels["Distance"].config(text=f"{distance} m")

labels["Direction"].config(text=direction, fg=dir_color)

labels["Speed"].config(text=f"{speed} km/h")

labels["SNR"].config(text=snr)

# Status indicator

arrow = "▶▶" if direction == "Approaching" else "◀◀"

status_var.set(

f"  {arrow}  Target {direction}  |  "

f"{distance} m  |  {speed} km/h"

)

# Push to buffers

dist_data.append(dist_f)

speed_data.append(speed_f)

snr_data.append(snr_f)

updated = True

if updated:

xdata = list(range(MAX_POINTS))

line_dist.set_data(xdata,  list(dist_data))

line_speed.set_data(xdata, list(speed_data))

line_snr.set_data(xdata,   list(snr_data))

canvas.draw_idle()

except Exception:

pass

return line_dist, line_speed, line_snr

# ─────────────────────────────────────────────

# RUN

# ─────────────────────────────────────────────

ani = FuncAnimation(fig, update, interval=100, blit=False)

root.mainloop()

Python Code :
import serial

import serial.tools.list_ports

import tkinter as tk

import matplotlib.pyplot as plt

import matplotlib.gridspec as gridspec

from matplotlib.backends.backend_tkagg import FigureCanvasTkAgg

from matplotlib.animation import FuncAnimation

import collections

import time

# ─── PORT ───────────────────────────────────────────

PORT = "/dev/radar"

BAUD = 115200

try:

ser = serial.Serial(PORT, BAUD, timeout=1)

print(f"Connected: {PORT}")

except:

ports = serial.tools.list_ports.comports()

ser = None

for p in ports:

if 'ACM' in p.device or 'USB' in p.device:

ser = serial.Serial(p.device, BAUD, timeout=1)

print(f"Fallback: {p.device}")

break

if ser is None:

print("ERROR: No port found!")

exit(1)

# ─── DATA BUFFERS ────────────────────────────────────

MAX_POINTS = 100

dist_data  = collections.deque([0]*MAX_POINTS, maxlen=MAX_POINTS)

speed_data = collections.deque([0]*MAX_POINTS, maxlen=MAX_POINTS)

snr_data   = collections.deque([0]*MAX_POINTS, maxlen=MAX_POINTS)

time_data  = collections.deque(range(MAX_POINTS), maxlen=MAX_POINTS)

t = [0]

# ─── WINDOW ──────────────────────────────────────────

root = tk.Tk()

root.title("HLK-LD2451 Radar")

root.configure(bg="#0a0a0a")

# Half screen left side

sw = root.winfo_screenwidth()

sh = root.winfo_screenheight()

w  = sw // 2

h  = sh

root.geometry(f"{w}x{h}+0+0")

# ─── INFO BOX (top quarter) ──────────────────────────

info_frame = tk.Frame(root, bg="#0a0a0a", pady=5)

info_frame.pack(fill="x", padx=10)

tk.Label(info_frame,

text="━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 10)).pack(anchor="w")

tk.Label(info_frame,

text="  HLK-LD2451 RADAR MONITOR",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 12, "bold")).pack(anchor="w")

tk.Label(info_frame,

text="━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 10)).pack(anchor="w")

labels = {}

field_config = [

("Alarm",     "#ffffff"),

("Angle",     "#00cfff"),

("Distance",  "#ffff00"),

("Direction", "#ff4444"),

("Speed",     "#ff9900"),

("SNR",       "#cc88ff"),

]

for field, color in field_config:

row = tk.Frame(info_frame, bg="#0a0a0a")

row.pack(anchor="w", pady=1)

tk.Label(row,

text=f"  {field:<12}: ",

bg="#0a0a0a", fg="#888888",

font=("Courier", 11)).pack(side="left")

labels[field] = tk.Label(row,

text="---",

bg="#0a0a0a", fg=color,

font=("Courier", 11, "bold"))

labels[field].pack(side="left")

tk.Label(info_frame,

text="━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━",

bg="#0a0a0a", fg="#00ff88",

font=("Courier", 10)).pack(anchor="w")

# status bar

status_var = tk.StringVar(value="Waiting for target...")

tk.Label(info_frame,

textvariable=status_var,

bg="#0a0a0a", fg="#888888",

font=("Courier", 9)).pack(anchor="w", pady=2)

# ─── GRAPH (bottom quarter) ──────────────────────────

fig = plt.Figure(figsize=(w/100, h*0.55/100), dpi=100)

fig.patch.set_facecolor("#0a0a0a")

gs = gridspec.GridSpec(3, 1, figure=fig,

hspace=0.6,

top=0.92, bottom=0.1)

# Distance plot

ax1 = fig.add_subplot(gs[0])

ax1.set_facecolor("#111111")

ax1.set_ylabel("Dist (m)", color="#ffff00", fontsize=7)

ax1.set_title("Distance", color="#ffff00", fontsize=8, pad=3)

ax1.tick_params(colors="#555555", labelsize=6)

ax1.set_ylim(0, 50)

ax1.grid(True, color="#222222", linewidth=0.5)

for sp in ax1.spines.values():

sp.set_color("#333333")

line_dist, = ax1.plot([], [], color="#ffff00",

linewidth=1.2, label="Distance")

ax1.fill_between(range(MAX_POINTS), 0, 0,

alpha=0.15, color="#ffff00")

# Speed plot

ax2 = fig.add_subplot(gs[1])

ax2.set_facecolor("#111111")

ax2.set_ylabel("Speed (km/h)", color="#ff9900", fontsize=7)

ax2.set_title("Speed", color="#ff9900", fontsize=8, pad=3)

ax2.tick_params(colors="#555555", labelsize=6)

ax2.set_ylim(0, 30)

ax2.grid(True, color="#222222", linewidth=0.5)

for sp in ax2.spines.values():

sp.set_color("#333333")

line_speed, = ax2.plot([], [], color="#ff9900",

linewidth=1.2)

# SNR plot

ax3 = fig.add_subplot(gs[2])

ax3.set_facecolor("#111111")

ax3.set_ylabel("SNR", color="#cc88ff", fontsize=7)

ax3.set_title("Signal Strength", color="#cc88ff", fontsize=8, pad=3)

ax3.tick_params(colors="#555555", labelsize=6)

ax3.set_ylim(0, 255)

ax3.grid(True, color="#222222", linewidth=0.5)

for sp in ax3.spines.values():

sp.set_color("#333333")

line_snr, = ax3.plot([], [], color="#cc88ff",

linewidth=1.2)

canvas = FigureCanvasTkAgg(fig, master=root)

canvas.get_tk_widget().pack(fill="both", expand=True, padx=5)

# ─── UPDATE FUNCTION ─────────────────────────────────

def update(_):

try:

while ser.in_waiting:

raw = ser.readline().decode(errors="ignore").strip()

parts = raw.split(",")

if len(parts) != 6:

continue

alarm, angle, distance, direction, speed, snr = parts

if int(alarm) > 1:

continue

# Update labels

dir_color = "#ff4444" if direction == "Approaching" else "#00ff88"

labels["Alarm"].config(text=alarm)

labels["Angle"].config(text=f"{angle} deg")

labels["Distance"].config(text=f"{distance} m")

labels["Direction"].config(text=direction, fg=dir_color)

labels["Speed"].config(text=f"{speed} km/h")

labels["SNR"].config(text=snr)

# Status

status_var.set(

f"Target detected  |  "

f"{'🔴' if direction=='Approaching' else '🟢'} "

f"{direction}"

)

# Update buffers

t[0] += 1

dist_data.append(float(distance))

speed_data.append(float(speed))

snr_data.append(float(snr))

time_data.append(t[0])

xdata = list(range(MAX_POINTS))

# Update plots

line_dist.set_data(xdata, list(dist_data))

line_speed.set_data(xdata, list(speed_data))

line_snr.set_data(xdata, list(snr_data))

for ax in [ax1, ax2, ax3]:

ax.set_xlim(0, MAX_POINTS)

canvas.draw_idle()

except Exception as e:

pass

return line_dist, line_speed, line_snr

ani = FuncAnimation(fig, update, interval=100, blit=False)

root.mainloop()


YOLO code 

import cv2

import numpy as np

import time

import threading

# ─── CONFIG ───────────────────────────────────────────

BLOB_SIZE   = 160

CONF_THRESH = 0.45

NMS_THRESH  = 0.4

CAM_W, CAM_H = 320, 240      # ← Camera resolution bhi kam ki

ALLOWED = {"person", "bicycle", "car", "motorbike", "bus", "truck", “drone”,”aeroplane”}

# ──────────────────────────────────────────────────────

net = cv2.dnn.readNet("yolov3-tiny.weights", "yolov3-tiny.cfg")

net.setPreferableBackend(cv2.dnn.DNN_BACKEND_OPENCV)

net.setPreferableTarget(cv2.dnn.DNN_TARGET_CPU)

with open("coco.names") as f:

classes = [l.strip() for l in f]

layer_names = net.getLayerNames()

out_layers  = [layer_names[i - 1] for i in net.getUnconnectedOutLayers().flatten()]

# ─── THREADED CAMERA ──────────────────────────────────

class CameraStream:

def __init__(self, src=0):

self.cap = cv2.VideoCapture(src)

self.cap.set(cv2.CAP_PROP_FRAME_WIDTH,  CAM_W)

self.cap.set(cv2.CAP_PROP_FRAME_HEIGHT, CAM_H)

self.cap.set(cv2.CAP_PROP_BUFFERSIZE, 1)

self.ret, self.frame = self.cap.read()

self.lock    = threading.Lock()

self.running = True

threading.Thread(target=self._update, daemon=True).start()

def _update(self):

while self.running:

ret, frame = self.cap.read()

with self.lock:

self.ret, self.frame = ret, frame

def read(self):

with self.lock:

return self.ret, self.frame.copy() if self.ret else (False, None)

def stop(self):

self.running = False

self.cap.release()

# ─── DETECT FUNCTION ──────────────────────────────────

def detect(frame):

h, w = frame.shape[:2]

blob = cv2.dnn.blobFromImage(frame, 1/255.0,

(BLOB_SIZE, BLOB_SIZE),

swapRB=True, crop=False)

net.setInput(blob)

outs = net.forward(out_layers)

boxes, confs, cids = [], [], []

for out in outs:

for det in out:

scores = det[5:]

cid  = int(np.argmax(scores))

conf = float(scores[cid])

if conf > CONF_THRESH and classes[cid] in ALLOWED:

cx, cy, bw, bh = det[:4]

x = int((cx - bw/2) * w)

y = int((cy - bh/2) * h)

boxes.append([x, y, int(bw*w), int(bh*h)])

confs.append(conf)

cids.append(cid)

idxs = cv2.dnn.NMSBoxes(boxes, confs, CONF_THRESH, NMS_THRESH)

results = []

if len(idxs) > 0:

for i in idxs.flatten():

results.append((boxes[i], classes[cids[i]], confs[i]))

return results

# ─── MAIN LOOP ────────────────────────────────────────

cam = CameraStream(0)

time.sleep(1)

detections  = []

fps_display = 0.0

# ← Ye sahi FPS measure karta hai (inference time)

infer_times = []

print(f"Running | Blob={BLOB_SIZE}x{BLOB_SIZE} | Press Q to quit")

while True:

ret, frame = cam.read()

if not ret:

break

# ── YOLO inference + sahi time measure ──

t1 = time.time()

detections = detect(frame)

t2 = time.time()

infer_ms = (t2 - t1) * 1000          # milliseconds

infer_times.append(t2 - t1)

if len(infer_times) > 20:

infer_times.pop(0)

# Real FPS = 1 / average inference time

real_fps = 1.0 / (sum(infer_times) / len(infer_times))

# ── Draw detections ──

for (x, y, bw, bh), label, conf in detections:

cv2.rectangle(frame, (x,y), (x+bw, y+bh), (0,255,0), 2)

cv2.putText(frame, f"{label} {conf:.2f}",

(x, y-8), cv2.FONT_HERSHEY_SIMPLEX, 0.5, (0,255,0), 1)

# ── Show REAL FPS + inference time ──

cv2.putText(frame, f"Real FPS : {real_fps:.1f}",

(10, 30), cv2.FONT_HERSHEY_SIMPLEX, 0.7, (0,0,255), 2)

cv2.putText(frame, f"Infer ms : {infer_ms:.0f}ms",

(10, 60), cv2.FONT_HERSHEY_SIMPLEX, 0.7, (255,128,0), 2)

cv2.imshow("YOLO Fast - Radxa Zero 3W", frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cam.stop()

cv2.destroyAllWindows()

print(f"Final Real FPS: {1.0/(sum(infer_times)/len(infer_times)):.1f}")